Please keep this document confidential

## General concept

- Gas production pushes the liquid in a U-tube to touch a liquid level sensor

- The liquid level sensor signals Arduino to trigger a solenoid valve to open to the atmosphere, which resets the liquid level

- Each liquid level change and reset is a consistent known volume. Counting the times that liquid level is triggered gives the gas production volume

Step 1:

Step 2:

Step 3:

Step 4:

## Arduino board draft

## Parts needed (some may be out of stock, need to find equivalents)

Must have:

- One level sensor ($25): Honeywell S&C - LLE101000 Adafruit: https://www.adafruit.com/product/3397

- One Solenoid Valve ($60): Asco 411M1312HV https://www.radwell.com/Buy/ASCO/ASCO/411M1312HV

- One Manifold ($28/pack of 4):  clippart #15490-5 http://www.clippard.com/

- [Already in lab] One #10-32 to 1/8” ID Hose Fitting ($6/Pack of 10): #11752-3-PKG http://www.clippard.com/

- One Arduino Uno R3 (Atmega328 - assembled) ($24) #50 https://www.adafruit.com/

- One RGB LCD Shield Kit w/ 16x2 Character Display - Only 2 pins used! - NEGATIVE DISPLAY ($24) #714 https://www.adafruit.com/

- One TIP120 Power Darlington Transistors - 3 pack ($2.5) #976 https://www.adafruit.com/

- One 12 VDC 1000mA regulated switching power adapter - UL listed ($8) #798 https://www.adafruit.com/

- 1N4001 Diode - 10 pack ($1.5) #755 https://www.adafruit.com/

- Adafruit Perma-Proto Quarter-sized Breadboard PCB - Single ($3) #1608 https://www.adafruit.com/

- 2.2K Ohm Flameproof Electronic Metal Film Resistor ($8.67) Amazon: https://www.amazon.com/gp/product/B016D8CAB4/ref=crt_ewc_title_dp_3?ie=UTF8&psc=1&smid=ATVPDKIKX0DER

Good to have but optional

- One Quick-Disconnect Push-In Signal/Power Connector - Male ($26): McMaster #6134T43 https://www.mcmaster.com

- One Quick-Disconnect Push-In Signal/Power Connector - Female ($14): McMaster #6134T33 https://www.mcmaster.com

- Small Plastic Project Enclosure - Weatherproof with Clear Top ($10) #903 https://www.adafruit.com/

## Arduino Codes

// LCD library includes

#include <Wire.h>

#include <Adafruit_MCP23017.h>

#include <Adafruit_RGBLCDShield.h>

// Port definition

#define OPTO_PIN 2

#define VALVE_PIN 9

// LCD variable

Adafruit_RGBLCDShield lcd = Adafruit_RGBLCDShield();

// Other variables

int counter = 0;

bool open_valve = false;

void setup() {

// Initialize pin mode

pinMode(OPTO_PIN, INPUT);

pinMode(VALVE_PIN, OUTPUT);

Serial.begin(9600);

// set up the LCD's number of columns and rows:

lcd.begin(16, 2);

lcd.setBacklight(0x7);

lcd.setCursor(0,0);

lcd.print("Bio-Gas Counter");

lcd.setCursor(0,1);

lcd.print(counter);

//Serial.println(counter);

}

void loop() {

lcd.setCursor(0, 1);

// Read buttons status

uint8_t buttons = lcd.readButtons();

if (buttons & BUTTON_SELECT)

{

counter = 0;

lcd.print(counter);

}

if (buttons & BUTTON_LEFT) {

digitalWrite(VALVE_PIN,HIGH);

} else {

digitalWrite(VALVE_PIN,LOW);

}

gas_counter();

if(open_valve)

{

digitalWrite(VALVE_PIN,HIGH);

delay(2000);

digitalWrite(VALVE_PIN,LOW);

open_valve = false;

}

}

void gas_counter()

{

if(!digitalRead(OPTO_PIN))

{

counter++;

lcd.print(counter);

Serial.println(counter);

open_valve = true;

}

}

Overall reactor design for a two-phase system (just an idea for an old project, not necessary what we are developing here at WSU)